BreakAway Communications, Inc.

Elmhurst, IL

Specifications For Radio Controlled

Motor Drive System

Overview:

This project will involve the construction of an electronics package that combines a full-duplex FM radio system with a radio control system capable of controlling a mobile platform.  This mobile platform is an aluminum chassis, approximately 12" wide, 12" deep and 17" tall.  It contains the electronic controls, two drive motors and a 12-volt battery (35 amps).  The electronic controls for the chassis need to fit in an aluminum enclosure measuring 12" x 10" x 3".  The platform will propel a fiberglass robot character that contains two speakers for audio output and an electret condenser microphone to pick up conversations of the audience for transmission to the operator.  It also contains motors for arm movement and other ancillary functions.

FM Radio System:

The communications portion of the system will allow the operator to simultaneously transmit to and receive audio from the mobile platform with the following specifications:

Operator’s End

Radio must be full duplex with broadcast quality audio, lightweight and easy to conceal.

Operators’ headset should be lightweight and inconspicuous with an easily accessible  on/off switch for the microphone.  The microphone in the headset should not pick up extraneous background noise which might be broadcast over the robots’ speakers.

Range should be upwards of 200'.

Radios should be low power consumption using rechargeable batteries with each battery lasting a minimum of three hours.

Radios should be on a frequency not commonly used to avoid interference.  A system with multiple frequencies would be preferable.

The transmitter and receiver should be in one package.  Commercially available units are acceptable.

Mobile Platform

It should include an amplifier to boost the audio level of the operator’s transmission to the robot.  This amplifier should be two channels, 20 watts per channel, capable of powering two 4" x 6" eight ohm speakers in the robot.  The volume control for this audio level should be on the radio in the mobile platform.

An amplifier in the mobile platform is needed to boost the audio level from the robot’s microphone (an electret condenser microphone) for transmission to the operator.  This microphone needs sufficient amplification to pick up conversations 4' - 6' from the robot.  The radio receiver at the operator’s end needs a volume control to adjust this level.

Options: Voice altering circuitry

Radio Control System:

The control system can be tone-based using a keypad to control the following functions:

1.  Will need to activate forward and reverse movement of the drive motors, both independently and together.

2.  Will need six auxiliary functions, four of which will control the up/down movement of two arms on the robot replica.

3. Detailed description of functions required: (all functions momentary on while holding button)

a. 1 Left motor forward

b. 3 Right motor forward

c. 2 Both motors forward

d. 4 Left motor reverse

e.6 Right motor reverse

f. 5 Both motors reverse

g. 9 Right arm up (always motor)

h. 8 Right arm down

i. # Left arm up

j. 0 Left arm down (always motor)

k. 7 Auxiliary (while button held - 12 power to a pin – makes head nod up and down – micro switch reverses direction)

l. * Auxiliary (while button held - play sound effects to come out of the robot)

Operator’s End

Keypad unit should be small enough to be operated in one hand.

Tone controls would ideally be sub-audible, transmitted over the voice radio system.  A separate radio to transmit the tone controls would be acceptable.  If a separate radio is used, it should have low power consumption with a range upwards of 200'.

The system needs to be designed so that only commands from the operators’ keypad will move the robot and it is not susceptible to movements from extraneous signals.

Mobile Platform

The circuitry should be capable of handling the following power consumption:

Drive Motors: 6 amps nominal, 12 - 15 amps peak.

Auxiliary Circuits (includes arm motors): 3 amps nominal, 6 - 8 amps peak.

Note:  A large margin of error should be factored into the current draw.

The circuitry which controls the drive motors should “ramp-up” the speed rather than simply turn the motor on and off, which would cause jerky movement.  When the tone is received to turn on a drive motor, it should accelerate to full speed in about one second.

Options: Variable speed, provided that the enclosure for the joystick control and switches for the auxiliary functions is not too large.  It must be easily controlled in one hand.

Version 1, Revised 2-20-08